Lab Report 5

ESE – 3014

EMBEDDED SYSTEMS COMMUNICATION PROTOCOLS AND SECURITY

Submitted to :

Linchan Wang

Submitted by:

Gurvinder Singh(748418)

1. Simulate the operations performed in the regeneration circuit of the PCM system receiver(like the figure below) include input, amplifier equalizer,timing circuit, decision making device, and output. You should add ap-propriate noise in distorted signal (in your input), and calculate the biterror rate at the end of receiver.

Solutoin:

>>

% PCM ENCODING

f = 5;

fs = 50;

Ts = 1/fs;

fss = 1.e4;

Tss = 1/fss;

t = 0:Tss:2-Tss;

d = Ts/40:Ts:2+Ts/40;

p = pulstran(t,d,&#39;rectpuls&#39;,1/(fs*40));

for i=1:length(em)

if ((((i&gt;1)&amp;&amp;(em(i)~=em(i-1)))||(i==1))&amp;&amp;(em(i)~=0))

x(j) = em(i)-1;

j=j+1;

end

end

% ENCODING MSG

j = 1;

z = dec2bin(x,5);

z = z&#39;;

z = z(:);

z = str2num(z);

s = 2*(z&#39;)-1;

Tb = 2/length(s);

fb = 0.5/Tb;

BL = Tb/Tss;

y = ones(BL,1);

h = 5*y*s;

h = h(:);

h = h&#39;;

% Analog MSG Signal

m = sin(2*pi*f*t)+1.1;

% Sampled signal

ms = m.*p;

% Quantized Msg

qm = imquantize(ms,2/16);

em = 8*(qm);

% PCM DE-CODING

rb = h(ceil(Tb/(Tss)):(Tb/Tss):length(h));

rb = (rb+5)/10;

l = length(rb);

for i = 1:l/5

q = rb((5*i)-4:5*i);

q = num2str(q);

x1(i) = bin2dec(q);

e(i) = x1(i)+1;

end

e = e/8;

y1 = ones(1,ceil((Ts/40)/Tss));

y2 = zeros(1,(Ts/Tss)-length(y1));

y3 = [y1 y2];

y3 = y3&#39;;

ms1 = y3*e;

ms1 = ms1(:);

% Filtering Sampled Signal

[n,w] = buttord(f/fss,(f+1)/fss,.6,4);

[a,b] = butter(n,w,&#39;low&#39;);

rm = filter(a,b,ms1);

rm = rm*50;

% Plotting Signals

figure(1);

subplot(2,1,1)

plot(t,m,&#39;b&#39;,t,ms,&#39;r&#39;);

legend(&#39;Analog Msg&#39;,&#39;Sampled Msg&#39;)

grid;

xlabel(&#39;t --&gt;&#39;);

ylabel(&#39;Amplitude&#39;);

axis([0 2 0 2.25]);

subplot(2,1,2)

plot(t,ms,&#39;k&#39;,t,qm,&#39;r&#39;);

legend(&#39;Sampled Msg&#39;,&#39;Quantized Msg&#39;)

grid;

xlabel(&#39;t&#39;);

ylabel(&#39;Amplitude&#39;);

axis([0 2 0 2.25]);

figure(2);

subplot(2,1,1)

plot(t,em,&#39;b&#39;)

xlabel(&#39;t&#39;);

ylabel(&#39;Amplitude&#39;);

title(&#39;Leveled Signal&#39;);

grid;

axis([0 2 -0.5 16.5]);

subplot(2,1,2)

plot(t,h,&#39;k&#39;)

xlabel(&#39;t&#39;);

ylabel(&#39;Amplitude&#39;);

title(&#39;POLAR NRZ Signal&#39;);

grid;

axis([0 2 -5.25 5.25]);

figure(3);

subplot(2,1,1)

plot(t,ms1,&#39;b&#39;);

title(&#39;Recovered Signal&#39;)

grid;

xlabel(&#39;t&#39;);

ylabel(&#39;Amplitude&#39;);

axis([0 2 0 2.25]);

subplot(2,1,2)

plot(t,rm,&#39;b&#39;);

title(&#39;Recovered Analog signal&#39;)

grid;

xlabel(&#39;t&#39;);

ylabel(&#39;Amplitude&#39;);

axis([0 2 0 2.25]);

In octave

>>PCM Wave

>>Timing Wave

>>Distorted Wave

>>Bit Rate Error

Note- Combined pictures are not clear so i am adding output pictures separately.

>>PCM wave

>>Timing Wave

>>Distorted Wave

>>Bit Rate